/*## Configure the SPI4 peripheral ###*/

Spi4Handle.Instance            	= SPI4;   //use STM32F429 SPI4

Spi4Handle.Init.Direction      	= SPI_DIRECTION_2LINES_RXONLY;

Spi4Handle.Init.CLKPhase       	= SPI_PHASE_1EDGE;    //read at DCLK falling edge

Spi4Handle.Init.CLKPolarity   	 = SPI_POLARITY_HIGH; //read at DCLK falling edge

Spi4Handle.Init.DataSize      	= SPI_DATASIZE_8BIT;  //or 16BIT

Spi4Handle.Init.NSS            	= SPI_NSS_HARD_INPUT; //make /CS low active

Spi4Handle.Init.Mode 		= SPI_MODE_SLAVE;     //MCU SPI4 as SPI Slave

/*## Enable EXTI0 and SPI4 to Receive AD7768 Data bits ###*/

// clear EXTI0 IT flag prior to enable external interrupt 0 !!!

__HAL_GPIO_EXTI_CLEAR_IT(KEY_BUTTON_PIN);

HAL_NVIC_EnableIRQ(EXTI0_IRQn);

// wait for EXTI0 interrupt (/DRDY rising edge) to prepare for reading last conversion data

if (EXTI0_Flag == SET)

{

EXTI0_Flag = RESET;//clear /DRDY rising edge flag variable

// throw out the last byte/word captured in the previous ODR cycle !!!

Rx_temp = *(__IO uint8_t *)&Spi4Handle.Instance->DR;		__HAL_SPI_ENABLE(&Spi4Handle);

// SPI4_CNVByteNum is the total data byte number to read in one conversion cycle

while (SPI4_ByteCount < SPI4_CNVByteNum)

{

// Check the RXNE flag

if (__HAL_SPI_GET_FLAG(&Spi4Handle, SPI_FLAG_RXNE))//

{

// transfer the received data from DR register to memory

SPI_RxBuffer[RxBuf_Idn] = *(__IO uint8_t *)&Spi4Handle.Instance->DR;

RxBuf_Idn++;

SPI4_ByteCount++;

}

}

// disable SPI4 to prevent read in extra data after all channel codes finished due to /DRDY is low active and DCLK continuously pulses

__HAL_SPI_DISABLE(&Spi4Handle);

SPI4_CNVCount++;

RxBuf_Idn = SPI4_CNVCount * SPI4_CNVByteNum;					SPI4_ByteCount = 0;

}//end of if (EXTI0_Flag == SET)

else

{//*** other software jobs ***//}

/*## handles External 0 interrupt request ###*/

// EXTI0 rising edge triggered to leave more response time for going into EXTI0_IRQHandler !!!

void EXTI0_IRQHandler(void)

{

if(__HAL_GPIO_EXTI_GET_IT(EXTI0) != RESET)

{

// enable SPI4 as soon as possible, and make sure before the first DCLK falling edge after /DRDY falling !!!

__HAL_SPI_ENABLE(&Spi4Handle);

__HAL_GPIO_EXTI_CLEAR_IT(EXTI0);

EXTI0_Flag = SET;

}

}