/*## Configure the SPI4 as Master and SPI5 as Slave ###*/

Spi4Handle.Init.Direction         	= SPI_DIRECTION_2LINES;

Spi4Handle.Init.CLKPhase          	= SPI_PHASE_1EDGE;//read at DCLK falling edge

Spi4Handle.Init.CLKPolarity      	 = SPI_POLARITY_HIGH;//read at DCLK falling edge

Spi4Handle.Init.DataSize        	= SPI_DATASIZE_16BIT;

Spi4Handle.Init.NSS               	= SPI_NSS_SOFT;// NSS pin is configured as GPIO output for /CS

Spi4Handle.Init.Mode 		= SPI_MODE_MASTER;// SPI4 as SPI Master

Spi5Handle.Init.Direction         	= SPI_DIRECTION_2LINES_RXONLY;// only receive data

Spi5Handle.Init.NSS               	= SPI_NSS_HARD_INPUT;

Spi5Handle.Init.Mode 		= SPI_MODE_SLAVE;// SPI5 as SPI Slave

/*## Enable SPI4 as Master and SPI5 as Slave to Receive AD7606B Codes ###*/

__HAL_SPI_ENABLE(&Spi4Handle);

__HAL_SPI_ENABLE(&Spi5Handle);

while (SPI4_CNVCount < SPI4_CNVNum)

{

CLR_CNV();

SET_CNV();//AD7606B conversion start

// wait for conversion finish, BUSY goes from high to low. Polling or interrupt mode

while (BUSY == SET) {;}

while (SPI4_WordCount < SPI4_CNVWordNum)// code number to read per conversion cycle

{

CLR_CS();

*(__IO uint8_t *)&Spi4Handle.Instance->DR = 0;

while (__HAL_SPI_GET_FLAG(&Spi4Handle, SPI_FLAG_RXNE) != SET); 				Delay_xus(1);// need half SCLK cycle delay for slow SCLK rate < 10MHz

SET_CS();

SPI_RxBuffer[RxBuf_Idn] = *(__IO uint16_t *)&Spi4Handle.Instance->DR;

SPI_RxBuffer[RxBuf_Idn+ADCSDO1_WordIdn] = *(__IO uint16_t \\ *)&Spi5Handle.Instance->DR;

RxBuf_Idn++;

SPI4_WordCount += 2;

}

SPI4_CNVCount++;

RxBuf_Idn = SPI4_CNVCount * SPI4_CNVWordNum;

SPI4_WordCount = 0;

}//while (SPI4_CNVCount < SPI4_CNVNum)

__HAL_SPI_DISABLE(&Spi4Handle);

__HAL_SPI_DISABLE(&Spi5Handle);